Fire extinguisher

Policy FHS 048P

Introduction

Prevention is the primary requirement of fire risk control, but the local provision of firefighting facilities is also vital in protecting people and property as the time delay in starting to deal with a small outbreak is critical in preventing it from developing into a major fire.

The Company shall use this document to guide provision and use of fire extinguishers, which are part of arrangements for reducing fire risks.

## Management and assessment for fire suppression systems

| Stage 1 | Assessments: conduct assessments of premises to identify and specify the needs for fire suppression systems. |
| --- | --- |
| Stage 2 | The result of inspection shall be recorded and retained, and suitable devices shall be specified designed and installed, tested, inspected, and thoroughly examined. |
| Stage 3 | All staff shall be told of fire suppression arrangements. Portable and fixed. |
| Sage 4 | Suitable and sufficient members of staff shall be trained in the use of fire suppression equipment. |
| Stage 5 | Monitor the performance. |
| Stage 6 | Audit. |
| Stage 7 | Management review. |

### Stage 1 – portable fire extinguishers

All Company premises shall be provided with the appropriate types and numbers of portable fire extinguishers to deal with the fire risk.

Fire extinguishers shall be provided in sufficient quantities and distributed so that nobody has to travel more than 30m in any direction to reach one.

Portable fire extinguishers shall be located in conspicuous positions on brackets or stands, so that people following an escape route can readily see them.

Where practicable, they shall be located in the same position on each floor, together with other fire safety features such as break glass call points, to create 'fire points'.

Fire extinguishers shall not be located in:

- rooms or corridors away from exits, unless they are necessary to cover a particular hazard;

- concealed positions behind doors;

- cupboards or deep recesses;

- positions where they may cause an obstruction to exit routes;

- positions where they may be damaged by the movement of furniture or equipment.

Fire extinguishers are normally coloured red, but the extinguishing medium may be indicated by a colour-coded label. Some older extinguishers may be fully colour-coded.

## Fixing of portable fire extinguishers

Portable fire extinguishers shall be fixed to the wall using the manufacturer’s brackets. Where walls are not substantial enough to retain fixing brackets, such as plasterboard walls, the extinguishers shall be sited on stands.

Heavier extinguishers shall be fixed to the wall with their handles about 1m from the floor.

Smaller extinguishers and fire blankets shall be fixed so that the handle of the extinguisher and the base of the fire blanket are approx. 1.5m above the floor.

## Use of portable fire extinguishers

The first priority when a fire is discovered is to raise the alarm so that evacuation of the building is initiated.  People in the vicinity of the outbreak should only attempt to extinguish the fire after this has been done and then only if they are confident that this can be accomplished without risk to their personal safety, and they have been trained in the use of portable fire extinguishers.

If this is the case, the following rules must be observed when tackling a fire:

- never use a portable extinguisher unless you are certain that it is the correct type to deal with the fire;

- only tackle a fire if there is a clear escape route behind you;

- never use more than one extinguisher. If the fire is not extinguished when the first extinguisher is exhausted, leave the building immediately and do not go in search of a replacement;

- direct the extinguishing medium at the base of the fire until the extinguisher is empty.

## Choice of fire extinguisher

Portable fire extinguishers are manufactured to deal with four main types of fire risk, as detailed in Table 1 and the Company shall select the correct fire extinguisher type(s).

| Class of material involved | Safe extinguishing media |
| --- | --- |
| Class A: fires involving wood, paper, fabrics, or other similar combustible material requiring cooling | Water Foam/AFFF Carbon dioxide Dry powder |
| Class B: fires involving flammable liquids or liquefiable solids (petrol, oils, greases, and fats) | Foam/AFFF Carbon dioxide Dry powder |
| Class C: fires involving gases or liquefied gases (butane, propane etc.) | Carbon dioxide Dry powder Warning: untrained people should not tackle fires involving gases contained in cylinders |
| Class D: metals | Warning: metal fires are difficult to extinguish using portable extinguishers and should not be tackled by untrained people |

Table 1

Note: electrical fires do not constitute a separate class. A fire involving electrical equipment falls into one of the four types of fire risk once the power supply has been disconnected. If it is not possible to disconnect the power supply, or there is doubt that the power supply has been disconnected, extinguishers with a non-conducting medium, such as carbon dioxide or dry powder (Class C), must be used.

The individual types of extinguishing medium, their method of action and the label colours assigned to them are described as follows:

RED – water

Use on Class A fires – cools and gives a limited smothering effect from steam formation.

CREAM – AFFF (aqueous film forming foam)

Use on Class A or B fires – extinguishes by smothering, creating a blanket of foam.

BLUE – dry powder

Use on Class A, B or C fires – works mainly by smothering as the powder forms a barrier to prevent air access. The powder also decomposes, a process which extracts heat, thereby assisting in extinguishing the fire by cooling.

BLACK	 – carbon dioxide

Use on Class A, B or C fires – works by smothering as it forms a dense non-flammable barrier around the fire which excludes oxygen.

Note 1:	metal fires require the application of a special extinguishing medium which is generally only available to the emergency services.

Note 2:	extinguishers used must be refilled / replaced at the earliest opportunity.

NEVER TACKLE A FIRE UNLESS YOU ARE SURE THAT YOU ARE USING THE CORRECT TYPE OF EXTINGUISHER AND THAT YOU HAVE MEANS OF ESCAPE. USE ONLY ONE EXTIGUISHER, AFTER WHICH ONCE EMPTIED MAKE YOUR ESCAPE IMMEDIATELY THROUGH REMAINING EXISTS.

## Stage 2 – inspection

The positioning of fire extinguishers shall be included in any building health and safety inspection.

Fire extinguishers shall be inspected and maintained by a competent person on an annual basis. Individual extinguishers shall be marked with the latest inspection date.

### Stage 3 – record-keeping

The Company shall keep records of all fire extinguisher maintenance.

The Company shall keep a register of employee training and appointment of an employee as competent to inspect and/or test fire extinguishers.

### Stage 4 – inform

The Company shall tell all employees on initial training and appointment of working of the fire detection and alarm systems along with the emergency evacuation and assembly arrangements.

### Stage 5 – monitoring

The Company shall monitor the performance of employees who are involved in the implementation of this policy.

### Stage 6 – audit

The Company shall audit to ensure that processes and documents relating to this policy are properly implemented.

### Stage 7 – review

The Company should review this policy as required by the outcome of monitoring, audit and/or changes in legislation or guidance.

Manager responsibilities

Managers shall ensure that adequate and appropriate fire extinguishers are correctly located in 
Company buildings.

They shall ensure that fire extinguisher equipment is checked inspected and tested by a competent person.

They shall ensure that all records are properly maintained.

They shall ensure that any reported defects or damage are dealt with.

Employee responsibilities

Employees are required to report to their manager:

- any visible damage or defects to fire extinguishers.

All employees must:

- not misuse fire extinguishers;

- use fire extinguishers only when it safe to do so, in compliance with their training and after raising 
the alarm.